SOLVENT SYSTEM DISPENSAL GUIDE-BULB

* READ AND UNDERSTAND DISPENSAL PROTOCOL BEFORE USING SYSTEM

* MISUSE WILL RESULT IN REVOCAL OF SOLVENT SYSTEM PRIVILEGES

PROTOCOL

- Bulb is full of old solvent

- Empty solvent by opening schlenk valve at the base of the bulb (Dispose into 4-liter waste receptacle on solvent system tray)

2    Backflush bulb 3 times with argon

- Turn A to left, pause, then back right (repeat 2 more times)

- Bulb now purged with argon and under vacuum

3.   Allow bulb to remain under vacuum until pump stops (~10 mbar)

4.   Turn C to left (closes line A)

5.   Turn A to left

6.   Turn B down

- Vacuum in bulb draws solvent into flask

* WARNING * Solvent fills very quickly

- Only withdraw the quantity of solvent you will use

7.   Turn B to left to stop solvent

8.   Turn C down

- Purges solvent in line and places bulb under argon

- The bulb must be stored in this position in order to maintain the purity of the solvent

# WITHDRAWING SOLVENT

- Bulb is full of fresh solvent

- Open upper schlenk valve

- Insert clean syringe needle or canula through septa and into desired solvent

- Withdraw desired quantity of solvent

- Close upper schlenk valve

* WARNING *  If septa contains 3 or more punctures, replace with new one

- Open upper schlenk valve

- Unscrew cap with used septa and replace with a new one

- Discard septa, but retain cap

- Close upper schlenk valve

** IF YOU HAVE ANY QUESTIONS OR ARE EVEN A LITTLE UNSURE ABOUT HOW TO USE THE SYSTEM, ASK TREVOR BEFORE INITIATING DISPENSAL **

These safety training resources, prepared solely for the use of the Regents of the University of California, were provided by a variety of sources.  It is your responsibility to customize the information to match your specific operations.  Neither the University of California nor any of its employees, makes any warranty, express or implied, or assumes any legal liability or responsibility for the accuracy, completeness, or usefulness of any information, or represents that its use would not infringe privately owned rights.  Reference herein to any specific commercial product, process, or service by trade name, trademark, manufacturer, or otherwise, does not necessarily constitute or imply its endorsement, recommendation, or favoring by the University of California. The views and opinions of authors expressed herein do not necessarily state or reflect those of the University of California, and shall not be used for advertising or product endorsement purposes.